|  | Mehran University of Engineering and Technology, Khairpur Department of Software Engineering |  |
| --- | --- | --- |

| Course: SWE-Computer Network Practical |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructor |  | Dr. Asad Raza Malik |  |  | Practical/Lab No. |  |  | 09 |  |  |
| Date |  | 08-10-2024 |  |  | CLOs |  |  | 03 |  |  |
| Student’s Roll no: |  |  |  |  | Point Scored: |  |  |  |  |  |
| Date of Conduct: |  | 08-10-2024 |  |  | Teacher’s Signature: |  |  |  |  |  |
| LAB PERFORMANCE INDICATOR | Subject Knowledge |  | Data Analysis and Interpretation | Ability to Conduct Experiment |  | Presentation | Calculation and Coding |  | Observation/ Result | Score |

| Topic | To configure the RIP dynamic routing protocol on two routers |
| --- | --- |
| Objective | - The objective of this lab is to configure the RIP (Routing Information Protocol) dynamic routing on two routers to enable communication between different network segments. - Students will learn how to set up and verify RIP version 2, understand its behaviour in sharing routing information, and analyze the effectiveness of RIP in maintaining accurate routing tables. - The lab aims to develop hands-on skills in implementing distance-vector routing protocols and troubleshooting basic connectivity issues in small to medium-sized networks. |

Lab Discussion: Theoretical concepts and Procedural steps

What is Routing Information Protocol (RIP)

RIP (Routing Information Protocol) is a distance-vector routing protocol that is used to determine the best path to a destination within a network. It operates based on the hop count, where the path with the fewest number of hops to the destination is selected. Here's some theoretical information about configuring RIP on two routers:

Key Concepts of RIP:

- Distance-Vector Routing Protocol: RIP uses hop count as its routing metric to determine the best path. A hop is defined as a passage through one router. The maximum hop count allowed for RIP is 15, which limits its use to smaller networks.

- Routing Updates: RIP routers send their entire routing table to their directly connected neighbors every 30 seconds. This periodic update can lead to higher bandwidth usage on larger networks.

- Versions of RIP:

  - RIP Version 1 (RIPv1): It is classful, which means it does not support subnet masks in its routing updates.

  - RIP Version 2 (RIPv2): It is classless and supports subnet masks, VLSM (Variable Length Subnet Masking), and multicast routing updates.

  - RIPng (RIP next generation): This version of RIP is used for IPv6 networks.

- Timers Used in RIP:

  - Update Timer: Every 30 seconds, the router sends out its entire routing table to all its neighbors.

  - Invalid Timer: If a route is not updated within 180 seconds, it is marked as invalid.

  - Flush Timer: If a route is not updated for 240 seconds, it is removed from the routing table.

  - Hold-down Timer: When a route is marked as unreachable, this timer prevents any changes to the route for a specific period, usually 180 seconds.

Implementation of RIP Routing in Cisco for Connecting Two Routers

Steps to Configure and Verify Two Router Connections in Cisco Packet Tracer:

Step 1: First, open the Cisco packet tracer desktop and select the devices given below:

| S. No | Device | Model Name | Qty. |
| --- | --- | --- | --- |
| 1. | PC | PC | 4 |
| 2. | Switch | PT-Switch | 2 |
| 3. | Router | PT-Router | 2 |

IP Addressing Table

| S. No | Device | IPv4 Address | Subnet-Mask | Default-Gateway |
| --- | --- | --- | --- | --- |
| 1. | PC0 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| 2. | PC1 | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| 3. | PC2 | 192.168.20.2 | 255.255.255.0 | 192.168.20.1 |
| 4. | PC3 | 192.168.20.3 | 255.255.255.0 | 192.168.20.1 |

- Then, create a network topology as shown below the image.

- Use an Automatic connecting cable to connect the devices with others.

Step 2: Configure the PCs (hosts) with IPv4 address and Subnet Mask according to the IP addressing table given above.

- To assign an IP address in PC0, click on PC0.

- Then, go to desktop and then IP configuration and there you will IPv4 configuration.

- Fill IPv4 address and subnet mask.

- Assigning IP address using the ipconfig command, or we can also assign an IP address with the help of a command.

- Go to the command terminal of the PC.

- Then, type ipconfig <IPv4 address><subnet mask><default gateway>(if needed)

- Repeat the same procedure with other PCs to configure them thoroughly.

Step 3: Configure router with IP address and subnet mask.

IP Addressing Table Router

| S. No | Device | Interface | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| 1. | Router 0 | FastEthernet0/0 | 192.168.10.1 | 255.255.255.0 |
|  |  | Serial 2/0 | 10.0.0.1 | 255.0.0.0 |
| 2. | Router 1 | FastEthernet0/0 | 192.168.20.1 | 255.255.255.0 |
|  |  | Serial 2/0 | 10.0.0.2 | 255.0.0.0 |

- To assign an IP address in router0, click on router0.

- Then, go to config and then Interfaces.

- Make sure to turn on the ports

- Then, configure the IP address in FastEthernet and serial ports according to IP addressing Table.

- Fill IPv4 address and subnet mask.

- Repeat the same procedure with other routers to configure them thoroughly.

Step 4: After configuring all of the devices we need to assign the routes to the routers.

To assign RIP routes to the particular router:

- First, click on router0 then Go to CLI.

- Then type the commands and IP information given below.

RIP Routes for Router0 are given below:

RIP Routes for Router1 are given below:

Step 5: Verifying the network by pinging the IP address of any PC.

we’ll use the ping command to do so.

- First, click on PC0 then Go to the command prompt

- then type ping <IP address of targeted node>

- as we can see in the below image we are getting replies which means the connection is working very fine

Assignment: Configuring RIP Dynamic Routing Protocol

Objective:

Configure RIP version 2 on two routers to enable communication between different network segments and verify the routing tables. Analyze how RIP dynamically updates routes and adapts to changes in the network topology.

Instructions:

- Network Setup:

  - Set up a network with two routers, Router A and Router B.

  - Connect Router A to a network segment (e.g., 192.168.1.0/24) and Router B to another network segment (e.g., 192.168.2.0/24).

  - Use a common link between the two routers, such as the 10.0.0.0/24 network.

- Configuration Tasks:

  - Assign appropriate IP addresses to all router interfaces.

  - Configure RIP version 2 on both Router A and Router B.

  - Specify the networks that should be advertised in the RIP protocol.

  - Save the configurations on both routers.

- Verification Tasks:

  - Use the show ip route command to view the routing tables on both routers.

  - Verify that the routes learned via RIP are present in the routing tables.

  - Use the debug ip rip command to monitor RIP update messages between the routers.

  - Simulate a network change (e.g., disconnect a link) and observe how RIP updates the routing tables.

- Analysis:

  - Analyze the behavior of RIP in terms of convergence time and how it responds to network changes.

  - Identify the limitations of RIP in terms of hop count and scalability.

  - Suggest scenarios where RIP would not be suitable and recommend alternative routing protocols for larger networks.

Submission Requirements:

- A screenshot of the routing table from each router.

- A brief report (1-2 pages) summarizing the configuration process, verification steps, and analysis of RIP's performance.

- Answers to the following questions:

  - How does RIP handle updates when a network link goes down?

  - What are the primary limitations of using RIP in modern networks?

  - How does RIP version 2 differ from RIP version 1?

Grading Criteria:

- Configuration (40 points): Correct setup and configuration of RIP on both routers.

- Verification (20 points): Proper use of commands to verify RIP operation.

- Analysis (30 points): Clear understanding of RIP's behavior and its limitations.

- Report Quality (10 points): Clarity, completeness, and organization of the report.

---END---